Nanocomposite Materials for Low Temperature VOCs Chemoresistors

Giuseppe Cappelletti*

Dipartimento di Chimica, Università degli Studi di Milano, via Golgi 19, 20133, Milano, Italy (giuseppe.cappelletti@unimi.it)

Abstract

The sensing of gas molecules is of fundamental importance for environmental monitoring, control of chemical processes, and non-invasive medical diagnostics based on human’s breath analysis [1,2]. In recent years, graphene-based gas sensors have attracted much attention and different materials have been developed [2]. However, they still suffer from several problems, which could be overcome by covering the graphene surface with metal oxides (MOS). Besides, thanks to the high chemical versatility, promising results could be also obtained by coupling porphyrin-based macrocycles to MOS. As such, boosted potentialities, especially in terms of tuned selectivity and low water interference, may be obtained. Therefore, the present work is aimed at evaluating and comparing the sensing at both mild temperatures (also exploiting the UV and solar light) of SnO2 matrix coupled with different porphyrins and graphene oxide materials towards the sensing of acetone molecules [4]. Specifically, zinc tetraphenylporphyrin (ZnTPP) and perfluorinated-modified ones were adopted. Switching the lamp on, the samples ability to sense acetone drastically changed: the LOD reached the 200 ppb for all the materials. A possible explanation of the role of both GO and porphyrins in boosting the SnO2 sensing of oxidizing molecules (as acetone) is reported, according to the recent literature related to hybrid chemoresistors [3,4] and DSSC devices [5].

[1]   A. Tricoli, N. Nasiri, S. De, Adv. Funct. Mater. 2017, 27, 1605271.

[2]   J. Chen, B. Yao, C. Li, G. Shi, Carbon 2013, 64, 225–229.

[3]   E. Pargoletti, U. H. Hossain, I. Di Bernardo, H. Chen, T. Tran-Phu, et al., Nanoscale 2019, 11, 22932

[4]   E. Pargoletti, U. H. Hossain, I. Di Bernardo, H. Chen, T. Tran-Phu, Get al., ACS App. Mat. and Interf. 2020, 12, 39549−39560

[5]   S. Berardi, St. Caramori, E. Benazzi, N. Zabini, A. Niorettini, A. Orbelli Biroli, M. Pizzotti, F. Tessore, G. Di Carlo, Appl. Sci. 2019, 9, 2739

Biography with photo

2016-present. Associate Professor, Dipartimento di Chimica, Università degli Studi di Milano, Milan, Italy. Scientific Research Activity: He is the co-author of 134 articles (H index = 34 citations: 3568, 16/05/22), all published on international journals with high impact factor, and more than 150 Communications in National and International Conferences. Research interests: electrocatalytic studies; composite materials for sensors in gas and liquid media. thin layers for medical applications; synthesis of ceramic materials; synthesis of nanomaterials for energetic applications and water/air pollutants remediation; superhydrophobic and superhydrophilic transparent layers; colloids and interfaces; industrial formulations.

Presenting author details 
Full name: Giuseppe Cappelletti
Contact number: +39 0250314228
Session name/ number: Nanomaterials & Nanocomposites
Presentation type: Oral